LW 352 3.0MEGAPIXEL HD NETWORK CAMERA

PART 2 – PRODUCTS

2.01	DESCRIPTION

- Multi-Codec (H.264, MJPEG) Streaming

- PoE (Power over Ethernet) Support

- 2 Power (DC 12V, PoE) Support

- Audio Support (G.711, G.726 Full Duplex)

- Pre/Post Alarm Support

- High resolution and high sensitivity with a 6.49 mm progressive scan CMOS.

- Auto White Balance

- Day & Night function

- WDR (Wide Dynamic Range) is available

- ABF Auto Back Focus function

2.02	GENERAL SPECIFICATIONS

- Image Device			6.49 mm Progressive Scan CMOS (3.0 Mega Pixels)

- Lens Type			DC-Iris, CS Mount

- Day & Night			ICR (IR Cut Removal)

- Min. Illumination			Color : 0.4 lx (F1.2, Gain : High)

B/W : 0.005 lx (F1.2, Gain : High)

- Image Enhancement		Backlight Compensation, 3D-DNR, Exposure Control, Sens

Up Control, AGC, White Balance, Privacy Masking, EIS,

Sharpness, Digital Zoom.

- Resolution (Maximum)		Up to 2040 x 1536

- Output				RJ-45 (Network), RCA (Installation)

- Frame Rate (Maximum)		30 fps @ 1920 x 1080, 1280 x 720

20 fps @ 2040 x 1536, 1280 x 1024

- Compression			H.264, MJPEG

- Multi-Streaming			Dual Streaming

- Audio In/Out			1/1

- Audio Compression		G.711, G.726

- 2-way Audio			Support

- Event				Alarm In/Out : 1/1

Motion detection : Support

Event Notification : Relay Out, Email, FTP

Pre Event Buffering : Support

- Integrated Client			LG Ipsolute VMS

- System Update			LG Ipsolute VMS or Web Client

- Connections			Up to 10

- Security				Password Protection, HTTPS(SSL, TLS)

- Ethernet				10/100 Ethernet

- Protocol

1.  IPv4 and IPv6			TCP/IP, UDP, HTTP, HTTPS, RTP, RTSP, DHCP, ICMP

2.  IPv4				FTP, SMTP, NTP, ARP, SNMP v1/v2/v3, DDNS(LG)

2.03	ELECTRICAL SPECIFICATIONS

A.  Power Source			PoE (IEEE 802.3af), DC 12V

B.  Input current			Maximum 580mA (DC 12V), Maximum 200mA(PoE)

2.04	ENVIRONMENTAL SPECIFICATIONS

A.  Operation Temperature/Humidity	-10°C to 50°C / 0% RH to 80 % RH

B.  Storage Temperature/Humidity	-20°C to 60°C / 0% RH to 85 % RH

2.05	MECHANICAL SPECIFICATIONS

A.  Dimension			57.2 mm x 68 mm x 149 mm

B.  Weight				508g

2.06	CERTIFICATIONS

A.  UL Listed

B.  CE Mark

C.  FCC Mark

D.	KCC